Rev:1

| 2017 COST of HULL MATERIALS - for W22 (hybrid) |  |  |
| --- | --- | --- |
| Description (WEIGHT) | Rough Cost | Quantity (main items only) |
| Plywood 4' x 8' (1.22 m x 2.44 m) x 6mm (main hull & cuddy) ~300 # | $1500 | - sheets ~ $3.00 / ft2 |
| Strip Cedar: ¾” x 3/8” (hull base) ~170 # | $1100 | 2200 feet 140 sqft @ 0.50/ft |
| Corecell 4' x 8' (1.22 m x 2.44 m) x 12mm (amas) ~70# | $1200 | 7 sheets |
| Spruce/Pine/Cedar (stringers) ~115# | $300 | 60 bd-ft (0.2m3) |
| Hardwood – mahogany 30# | $80 | 10 bd-ft (0.03 m³) |
| Epoxy resin & hardener, fillers 120# | $1100 | 11 gallons - depending on sheathing |
| Fibreglass cloth (6-oz) 70# | $700 | 200 sq-yd - depending on sheathing |
| Fibreglass - 45/45 Bias and Unidirectional cloth 50# | $400 | 90 sq.yd |
| Paint - Varnish to finish 55# | $400 | 4 galls |

Gross sub total:     980#        $ 6780  (incl. 7-10% scrap)

To which one adds: fittings, rigging, beams, mast, foils, sails – see below

ROUGH  TOTAL  COST:   (March 2017)

Basic material list    $6780  (see above list + $220 misc)

Fittings	          $2000

Sails                        $2500

Rigging                     $500

Mast-Boom             $2000

Equip                        $500

Ama Tubes               $500

Trampolines              $800

Misc                          $500

TOTAL:     $16,300

UPDATED BASIC INFO for STUDY PROFILE

Basic DESIGN DATA:  
     Displacement at DWL: 1900 lbs
     Buoyancy of Ama:   1700 lbs
     Estimated total weight:   ±545kg (1300 lbs)
     Estimated construction time:   900 -1100 hrs
     Estimated cost of materials:   $15000–$18000  . 
          depending on source and quality

(Estimated total material cost for full foam composite W22 = $22K) 2017